INDUCTION COOKER BLOCK DIAGRAM

DC INPUT (Battery)

48V @ 40A

Pot surface measured using an IR temperature sensor

Load (pot)

Magnetic field

Work coil

Voltage, measured from the same point as current, using a voltage divider

Resonant Tank (L + C total)

Power Inverter (full bridge; four MOSFETs) IRFP4110

Current measured from inverter output that flows through the coil using a shunt resistor

ADC inputs to MCU

- Heatsink Temperature feedback

- Coil temperature feedback

- Phase detection and calculation circuit

- LCD + Touch interface

- WIFI module (Web/Mobile App

MCU

Voltage, measured from 48V DC BUS input

Gate Drive

IRS21876 (2)

DC BUS (bulk capacitors)

Input filter, EMI filter, transient suppressor, fuse

## SYSTEM SPECIFICATION

Power Specification

- Input Power: 48V DC at 40A (1920W maximum)

- Topology: Full bridge inverter configuration

- Gate Driver: IRS21867 (x2)

- Power MOSFETs: IRFP4110 (100v, 120A, RDS (on) = 0.012Ω) (x4)

Control and Interface:

- Primary MCU with multiple feedback loops

- LCD display with capacitive touch interface

- Temperature slider for continuous adjustment

- IR temperature sensor for cookware temperature monitoring

- Wed App and Mobile app for remote control

Advanced Features:

- Automatic cookware placement detection

- Automatic cookware type detection (Material identification)

- Automatic cookware size detection

- Preset temperature modes (Boiling, Frying, warming etc.)

- Optimal temperature maintenance across different cookware

## DETAILED FEEDBACK SIGNALS THE MCU RECEIVES

1. Current Feedback

- High precision current shunt resistor

- Or Hall-effect sensor

What MCU learn from it:

- Actual coil current (RMS and peak)

- Real power delivered to cookware

- Over current / short-circuit detection

- Cookware material and loading characteristics (changes current waveform)

Critical Dependencies:

- Pot size (larger = higher current)

- Pot material (ferromagnetic response)

- Pan temperature (resistance changes)

- Load condition (empty vs. loaded)

- Cooking mode (warm, simmer, etc.)

2. Coil / Resonant Voltage Feedback

Where it comes from:

- Voltage divider connected to the resonant tank (coil + capacitor)

What the MCU learns from it:

- Tank voltage amplitude

- Resonant behavior and frequency shift

- Zero-crossing detection

- Cookware presence/removal detection

- Overvoltage protection triggers

3. DC Bus Voltage Feedback

Where it comes from

- Voltage divider on the main DC bus (e.g., the 12-48v powering the cooker)

What the MCU learns from it:

- Supply voltage stability

- Detect brownout

- Protect inverter from undervoltage/overvoltage

- Adjust power based on available DC supply

4 Temperature Feedback (multiple sensors)

(A) Heatsink Temperature

Source

- NTC thermistor mounted on MOSFET/ICBT heatsink

- Or digital sensor

Purpose:

- Power stage overheating prevention

- Automatic power reductio or shutdown

(B) Coil Temperature

Source

- NTC thermistor near the work coil

Purpose

- Glass cooktop overheat protection

- PCB thermal management

(C) Cookware Surface Temperature (IR Sensor – PRIMARY)

Source:

- MLX90614 or MLX90640 IR temperature sensor

- Mounted below the glass, pointing upward at the cookware bottom

Purpose:

- Maintain optimal cooking temperature

- Closed-loop temperature control for different cookware materials

- Preset temperature accuracy

- Compensate for different cookware thermal properties

- Prevent overheating and burning

Critical Function:

- The IR sensor provides real-time cookware temperature

- MCU adjusts power output to maintain setpoint

- Different cookware (aluminum, stainless steel, cast iron) heats differently

- Sensor compensates for these differences automatically

5. Pot Detection Feedback

Detection Method (Multi-Parameter Analysis):

(A) Placement Detection

- Rapid change in resonant frequency when cookware placed

- Sharp increase in coil current

- Phase angle shift detection

- Tank voltage amplitude change

(B) Type Detection (Material)

- Ferrous Steel: High current draw, low phase angle (5 - 15°)

- Stainless Steel (18/10): Lower current, moderate phase angle (15 - 30°)

- Cast Iron: Very high current, minimal phase angle (<10°)

- Aluminum (with ferromagnetic base): Moderate current, higher phase angle

(C) Size Detection

- Current amplitude correlation (larger = higher current)

- Resonant frequency shift magnitude

- Power transfer efficiency measurement

- Small: <18cm diameter, Current < 15A

- Medium: 18-24cm diameter, Current 15 – 25A

- Large: >24cm diameter, Current >25A

6. Phase Angle Feedback (Voltage vs Current Phase)

Source

- Simultaneous ADC sampling of voltage and current waveforms

- Software calculation of phase difference

MCU learns:

- Resonance condition (phase approximately 0° at resonance)

- Real power vs. reactive power

- Load heaviness (thermal mass)

- Cookware type identification

- Optimal switching frequency

Critical for:

- Frequency tuning (typically 20-40kHz)

- Cookware detection algorithm

- Efficiency optimization

7. User Interface Feedback

Physical Interface:

- Capacitive touch buttons (power, mode, timer)

- Capacitive slider for temperature adjustment (0-100%)

- LCD display (temperature, power lever, timer, mode)

Digital Interface:

- Web application (WIFI connectivity)

- Mobile app (WIFI)

- Remote monitoring and control

- Recipe preset and scheduling

MCU learns:

- Selected cooking mode

- Target temperature/power level

- Timer settings

- User preferences

Optional Advanced Feedbacks (future enhancements)

Pan temperature via IR sensor -> used for smart frying

Glass surface temperature sensor -> Protects user and electronics

## PRESET TEMPERATURE MODES

Mode Definitions

1. Warm Mode 60-70°C (140-160°F)

- Low power, gentle heating

- Keep food warm without cooking

2. Simmer Mode: 85-95°C (185-200°F)

- Low-medium power

- Gentle boiling for soups/sauces

3. Boil Mode: 100°C (212°F)

- High power to reach boiling

- Maintain rolling boil

4. Fry Mode: 160-190°C (320-375°F)

- Precise temperature control

- IR sensor maintains oil temperature

- Prevents burning/smoking

5. Sear Mode: 200-230°C (390-450°F)

- Maximum power initially

- High-temp maintenance for browning

6. Custom Mode: User-defined temperature

- Slider input or app control

- IR sensor maintains exact setpoint

## SAFETY FEATURES

Protection Mechanisms

- Overcurrent Protection: > 45A shutdown

- Overvoltage protection: DC bus >55V

- Undervoltage Protection: DC bus <40V

- Thermal Shutdown: Heatsink >90 °C

- No-Pot Detection: Shutdown if no cookware after 30s

- Wrong-Pot Detection: Alert if non-compatible cookware

- Overheat Protection: IR sensor > 300 °C emergency stop

- Short Circuit Protection: Instantaneous shutdown

User Safety

- Auto-shutoff timer (default 2 hours)

- Child lock feature

- Residual head indicator (glass temp > 50 °C)

- Automatic power reduction on overheating

## COMMUNICATION ARCHITECTURE

Local interface

Remote Interface

- WIFI module: ESP32

- Protocol: MQTT or HTTP REST API

- Features:

  - Real-time temperature monitoring

  - Remote Power/mode control

  - Timer scheduling

  - Cooking history logs

  - Recipe presets sync

App Features

- Temperature graphs

- Cooking notifications

- Multi-language support

- Firmware OTA updates

- Energy consumption tracking