## INDUCTION COOKER SYSTEM – LAYER-BY-LAYER BLOCK DIAGRAM

This diagram describes the physical, electrical, sensing, control, and communication layers of the induction cooker from top (cookware) to bottom (power input), showing how energy, signals, and feedback flow through the system.

## 1. COOKWARE & HEATING INTERACTION LAYER (TOPMOST)

Cookware (Pot / Pan)

- Ferromagnetic base (cast iron, steel, induction-compatible stainless steel)

- Eddy currents induced by an alternating magnetic field

- Direct heat generation inside the cookware material

Primary Phenomena

- Eddy current losses (I²R heating)

- Magnetic hysteresis losses

- Cookware temperature rise

## 2. OPTICAL & THERMAL SENSING LAYER (BELOW COOKWARE)

IR Temperature Sensor (MLX90614)

- Mounted below the glass, facing upward

- Measures cookware bottom surface temperature (non-contact)

- Provides real-time thermal feedback to the MCU

Functions

- Closed-loop temperature control

- Preset mode regulation (boil, fry, warm, etc.)

- Over-temperature protection (>300 °C emergency stop)

## 3. MECHANICAL ISOLATION & USER INTERFACE LAYER

Glass Cooktop (Ceramic / Tempered Glass)

- Electrically insulating

- Thermally resistant

- Mechanically supports cookware

- Transparent to IR radiation

User-Safety Role

- Electrical isolation between the user and power electronics

- Residual heat indication logic tied to temperature sensors

## 4. ELECTROMAGNETIC ENERGY TRANSFER LAYER

Work Coil (Induction Coil)

- High-current Litz wire

- Generates high-frequency alternating magnetic field (20–40 kHz)

Magnetic Field Coupling

- Field penetrates glass and cookware base

- Strength depends on coil current, frequency, and pot size

## 5. RESONANT POWER STAGE LAYER

Resonant Tank (L + C)

- Induction coil (L)

- Resonant capacitors (C)

- Tuned close to resonant frequency

Functions

- Maximizes power transfer efficiency

- Minimizes reactive current at resonance

- Enables phase-based cookware detection

Measured Signals

- Tank voltage (via voltage divider)

- Phase relationship with current

## 6. POWER CONVERSION & SWITCHING LAYER

Full-Bridge Inverter

- 4 × IRFP4110 MOSFETs

- Converts DC bus power into high-frequency AC

Gate Driver Stage

- IRS21867 / IRS21876 (×2)

- High-side & low-side drive

- Dead-time control and level shifting

Protection Role

- Fast shutdown on overcurrent, undervoltage, or fault

## 7. POWER MEASUREMENT & FEEDBACK LAYER

Current Sensing

- Shunt resistor or Hall-effect sensor

- Measures inverter output/coil current

Voltage Sensing

- Resonant tank voltage divider

- DC bus voltage divider (48 V input)

Derived MCU Information

- Real power vs reactive power

- Resonance condition (phase ≈ 0°)

- Cookware size, material, and presence

- Overcurrent / short-circuit detection

## 8. THERMAL MONITORING LAYER (ELECTRONICS)

Heatsink Temperature Sensor

- NTC thermistor or digital sensor

- Mounted on MOSFET heatsink

Coil Temperature Sensor

- NTC near induction coil

- Protects glass and PCB

Functions

- Thermal derating

- Automatic shutdown on overheat

## 9. CONTROL & INTELLIGENCE LAYER (CORE LOGIC)

Main MCU

Inputs

- Coil current

- Tank voltage

- DC bus voltage

- Phase angle (V–I)

- IR cookware temperature

- Coil temperature

- Heatsink temperature

- User commands (touch/app)

Processing Tasks

- Frequency tuning (20–40 kHz)

- Power modulation (PWM / frequency shift)

- Closed-loop temperature control

- Cookware detection (placement, size, type)

- Safety monitoring and fault handling

Outputs

- Gate driver control signals

- LCD/UI updates

- WiFi data packets

## 10. USER INTERFACE LAYER

Local Interface

- LCD display

- Capacitive touch buttons

- Temperature slider

Displayed Information

- Current temperature

- Mode (boil, fry, warm, etc.)

- Timer

- Fault messages

## 11. COMMUNICATION & CONNECTIVITY LAYER

WiFi Module (ESP32)

- MQTT or HTTP REST

- OTA firmware updates

Remote Interfaces

- Web application

- Mobile application

Capabilities

- Remote power control

- Temperature monitoring

- Preset selection

- Scheduling and notifications

## 12. POWER INPUT & PROTECTION LAYER (BOTTOMMOST)

DC Power Source

- 48 V Battery input

- Up to 40 A (≈1920 W)

Protection & Conditioning

- Fuse

- Transient suppressor (TVS)

- EMI filter

- Input filter

DC Bus

- Bulk capacitors

- Stable energy reservoir for inverter

## 13. OVERALL TOP-TO-BOTTOM FLOW SUMMARY

Cookware
↓
IR Temperature Sensing
↓
Glass Cooktop
↓
Magnetic Field
↓
Work Coil
↓
Resonant Tank (L+C)
↓
Full-Bridge Inverter
↓
Gate Drivers
↓
MCU Control & Feedback Processing
↓
DC Bus Capacitors
↓
EMI / Protection Network
↓
48 V DC Power Source

oo